# MDU Market Opportunity

Brownfield discussion document prepared for Forrest Reynolds

| Author: | Charles (Chuck) Siemonsma Email: charles.siemonsma@interactsolution.net Skype: CBSiemonsma Mobile: +1 605-351-9159 |
| --- | --- |
| Date: | September 17, 2022 |

# Contents

Synopsis	3

The Opportunity	4

Background	6

About Interact	8

Interact’s Objective	8

Interact’s TV Platform Strategy	8

Broadcast	8

Delivery	9

LAN (Local Area Network) Transport	9

IPTV Management	10

Managed Network and Personal Area Network (PAN)	10

MDU Owner Benefits:	11

Resident Benefits:	11

Interact Advantages	12

G.hn – IP over Copper/Coax (IPoC)	13

Applications	14

Distance limitations	14

A Deeper Technology Review	15

About G.hn Technology	15

UTP compatible cable types	15

Coaxial compatible cable types	15

Advantages over G.fast	16

Advantages over DOCSIS	16

Advantages over MoCa	16

# Synopsis

Interact has identified an opportunity to provide gigabit Internet and cutting-edge digital media streaming technologies and IPTV into the multi-dwelling/multi-resident, planned unit development and condominiums (referred throughout this document as the MDU markets) and out-of-home markets.

Broadband from cable companies and the incumbent Internet service providers (ISPs) is no longer a source of revenue for MDU owners.  Traditional Cable, Satellite and ISPs are either discontinuing video services or supplying antiquated and outdated cable television products and service offerings.  Interact is uniquely positioned to address these issues and views them as an incredible business opportunity.

There is a need to provide ‘skinny bundles’ (a more targeted number of channels, available to consumers, to purchase at a reduced price versus what they pay for traditional cable television) that offers local programming, cable news, sports and other targeted programming that are provided in a very cost-effective manner.

Interact has formed strategic partnerships with best-of-breed companies, integrating state-of-the-art networking, IPTV streaming, cloud-content provisioning and network management services.  The combination results in a best-of-breed virtual personal area network, streaming digital media and IoT managed solution.  The end-result is cost and service benefits to MDU owners and their residents by leveraging their existing (telephone and/or coax) wiring infrastructure at a highly competitive price point.

MDU owners will benefit from being able to provide high-speed gigabit Internet effectively creating a 4th utility for residents.  Combining this with corresponding digital media ‘skinny bundles’, Interact is bringing back the opportunity to monetize an asset that has been mismanaged by current providers which was once a source of revenue – now a cost center.

MDU owners also benefit from the intangible benefits of upgrading their technology implementations.  A recent report from market research firm RVA LLC. indicates that high-speed broadband, which usually means (up to) gigabit services, adds 2 percent to the value of a condominium and 8 percent to the rental price of an apartment.

Residents will also benefit from gigabit-enabled HSIA (high-speed Internet access), state-of-the-art building-wide wireless PAN (Personal Area Network) that supports all of the multi-device demands of the modern connected home, a superior linear high-definition television service providing access to over 250+ channels, catch-up TV, DVR (digital video recording) and VOD services, and by virtue of the high-speed Internet, access to all of their SVOD (subscription video on-demand) streaming media services.  Interact’s promise is to provide a superior resident experience, more programming at a compelling price.

# The Opportunity

The advent of OTT (Over the Top of the Internet) and IoT (Internet of Things) is creating a multitude of issues in the multi-dwelling/multi-tenant (MDU) space.  Once, it was enough to provide residents with a proprietary set-top-box, a single router and a Wi-Fi access point connecting three or four devices, such as a laptop, printer, phone and streaming media device. Today, a resident may want to connect twenty devices or more, all with their own set of unique challenges and requiring substantially higher bandwidth to accommodate their media streaming services.

### Current Providers – Shortcomings

A growing number of Cable Television/Multi-System Operators (MSOs) and Internet Service Providers (ISPs) have all but abandoned providing digital media and are now focusing solely on providing high-speed Internet access. They haven’t been able to figure out how to compete with the OTT, virtual multiple video programming distributors (vMVPD) and subscription video on-demand (SVOD) providers who include streaming services such as Netflix, Hulu, CBS Now, ESPN Now, HBO Go/HBO MAX, PhiloTV, AppleTV, Locast, ATT TV, Sling TV and a myriad of others.

Legacy cable and satellite subscriptions are on a steep decline with some MDUs in urban tech centers garnering just 7% to 8% adoption of services.  OTT subscriptions account for more than 50% of American households, with millennials at the forefront of this transition.  These subscriptions may include multiple streaming devices like AppleTV, Amazon Fire Stick, Roku Stick and others. Just as MDU owners are challenged with residents purchasing electric vehicles that require metered charging stations, MDU owners have also become burdened with challenges associated with poor Internet, antiquated old legacy cable services and the ever-increasing number of connected OTT and IoT devices being connected in a resident's apartment or condominium.

A recent article from Broadband Communities Magazine focused on why MDU owners need to make access to gigabit Internet a priority.

Traditionally, MDU owners would allow the cable MSO or LEC (local exchange carrier) / Telecom to install wiring in their buildings in order to be able to offer telephone, cable television and Internet.  Typically, the building owner would contract with two or three vendors who would compete for the opportunity to sell their services to the MDU’s residents.  Those vendors would then allow residents to decide which services they wanted and utilize the proprietary network infrastructure to provide those services. For this opportunity, these vendors would pay upfront infrastructure costs and even pay a small fee back to the MDU owner. That has all but dried up.  These companies now charge for the upfront capital expenditures as well as providing no future recurring revenues back to the MDU owners.

### Internet of Things (IoT)

Connected devices can include computers, smart phones, tablets, televisions, thermostats, door locks, voice control (Amazon Alexa/Google Home etc.) devices, speakers, security cameras, CO2 sensors, noise sensors, hue lighting, water sensors, and/or refrigerators.  Some millennials have over 60 connected devices in their home!

The major issue with IoT devices is that they are constructed with older 2.4GHz wireless chipsets (due to cost) that have impacts on Wi-Fi performance and create significant challenges when adding multiple devices, on top of the requirement for high-bandwidth streaming services.  That in itself is a challenge in a single-family home. But now, add this issue to the collective issues of residents above you, next to you and below you.  Not only do you have competing Wi-Fi access points creating interference, but you also have all of their IoT devices stepping on available Wi-Fi channels and creating significant interference, as well.

A recent survey, by the National Multi-Family Housing Council (NMHC) focused on the impact of smart phone IoT devices, interest level and impact on renting decisions.  Here is a snapshot from that survey:

The advent of OTT and IoT will continue to explode and the resulting impact to MDU owner/operators will be a requirement to remedy their network deficiencies.  Interact is uniquely qualified to address these issues and sees this as an incredible opportunity to positively affect the MDU Industry and the resident quality of experience.

# Background

We are living in unprecedented times with the impact of COVID-19 with the accelerated adoption of ‘work from home’ and ‘remote learning’.  Suddenly, the location of the ideal MDU is no longer the commute time to the office but, rather, whether it is served by gigabit Internet speed or not.  This is not just a small bump in the road – this will have a lasting impact on the MDU Industry and the way that residents live and work.

The MDU Industry will need to differentiate themselves in a competitive market to accommodate for this shift.  The consistency, speed and quality of technology being offered have become critical factors in capturing residents.

Take for example high-speed Internet access; residents used to work directly with Cable or Internet Service Providers (ISPs) to secure bandwidth they would use for their laptop, printer, tablet and/or smart phone. This eventually evolved into a multitude of connected devices, including streaming media devices, IoT devices, security cameras etc.  It’s clear, based on available information, surveys and the services being provided by current providers, that it is yet again time for a change – a shift to managed Internet, digital media and connected devices.  A move that will ultimately benefit residents and MDU owners alike.

This same survey outlined above, by the National Multi-Family Housing Council (NMHC), found that high-speed Internet is important or very important to 91.7% of apartment dwellers. Further, pre-installed Internet access is important to 74.8% of respondents. At the time of the study, 41.5% of residents in MDUs were telecommuting; a number that is sure to increase as we move on from COVID-19.

Here is another snapshot from that survey:

In the past 18-24 months, another phenomenon known as ‘cord cutting’ (disconnecting cable/satellite television services) has accelerated to over 33 million cord cutters (as of the end of 2018), with projections of 55.1 million cord cutters by 2022 in the United States.  Over this time, cable TV and satellite TV providers have lost millions of subscribers (nearly 2.5M in Q4 2019 alone) with estimates of an additional 26% decline in subscribers by 2030 over 31.3M households.

With the most recent quarterly numbers, we believe that these 2018 statistics grossly underestimate their actual predicted decline in subscribers.  It is our belief that this phenomenon is occurring at a much greater pace in the MDU Industry as millennials, those who are likely renting apartments, are the vast majority of ‘cord cutters’ looking for streaming media services.

These end-users have literally ‘cut the cord’ from the conventional TV service providers and opted to find their entertainment content via virtual multichannel video programming distributors (vMVPDs) via a free or subscription model:  Amazon Prime (40M active users), Roku (39.8M active users), AppleTV (33.6 active users), Hulu (31.8M active users), Chromecast (over 55M sold), YouTube, YouTube TV, Netflix, Sling TV, DirecTV Now, etc.   Combined, the vMVPDs have over 150M active users.

Recently, AT&T announced to their nearly 3,000 vendor dealers that they were no longer going to market DirecTV satellite (with an exception being rural markets) and that their existing DirecTV satellite implementations in various MDU and Out-of-Home markets would now be subject to competition from AT&T’s newly released vMVPD streaming solution called ‘AT&T TV’.

Some key points to consider when evaluating this new landscape:

Cable companies and ISPs are no longer subsidizing implementations within the MDU industry; nor are they providing any upsell revenues back to the MDU owner;

MDU integrator revenues have been decimated as a result of cord cutting and AT&T’s recent decision (these integrators were getting ~15% of gross revenues);

Most vendor/dealers are extremely disgruntled with AT&T and are actively searching for solutions that could replace AT&T/DirecTV with another proprietary digital media service;

Most integrators lack solid IP technical knowledge or background – they understand the basics but need assistance when migrating everything to an IP infrastructure;

These integrators have solid relationships with MDU owners and other out-of-home markets – having been the ‘feet-on-the-street’ for DirecTV, Dish Network and ISPs for over 25 years;

Most of the OTT/vMVPD service providers provide limited line-ups that may not include local programming, sports, cable news and other niche content – MFNs (most favored nations) and minimum revenue guarantees prohibit the delivery of that content via OTT providers;

vMVPD service providers promised low monthly recurring fees

  - Several providers, including YouTube TV and Hulu Live have undergone major price increases;

Most of the OTT/vMVPD providers do not provide ‘catch-up’ television or DVR service for linear programming – although most do provide access to SVOD libraries;

Internet bandwidth is increasingly challenged because of both WAN bandwidth availability and MDU local area network infrastructures;

Most are not aware of newer IP over Coax/UTP (unshielded twisted pair) switch gear, or refuse to consider its viability, that would garner cost-effective gigabit Ethernet transport.

Why is this important to MDU owners?  The MDU Industry has lost control of its asset; it no longer gets network infrastructure installed for free (or at a greatly subsidized rate); it no longer has a revenue stream from these vendors; and it likely is challenged with ever-increasing bandwidth requirements to support new streaming media services.  MDU owners ultimately will have to pay additional monies to provide HSIA, and to provide residents with reliable gigabit network transport technologies in order to support the ever-increasing demands for bandwidth.

# About Interact

Interact is an IPTV (Internet protocol television) distribution and technology company. Conceived and developed by seasoned hospitality technology executives, Interact’s mission is to simplify the end-user experience and offer the Multifamily (MDU/MTU, Condominiums and planned unit developments (PUDs)) and out-of-home markets (hospitality, senior care, hospital etc.) a way to empower their residents, guests and end-users. Interact provides users with the ability to access a multitude of digital media including linear broadcast television, subscription video-on-demand, on-demand content and integration with private repository and users’ social media subscriptions.

## Interact’s Objective

Provide cost-effective gigabit Internet access to the multi-dwelling building and distributing gigabit bandwidth inside the building, along with providing a better, more economical, linear programming package, will allow Interact the ability to help ISPs, dealers and integrators compete against vMVPD providers like AT&T TV. This will still allow residents to move to their Netflix, Amazon Prime, HBO Max subscriptions, but utilize Interact to fill the void on programing not readily available via OTT vMVPD subscriptions as well as engineer, install and support a gigabit Ethernet network

## Interact’s TV Platform Strategy

Interact’s turn-key solution is based on proven products and services, well integrated, to assure a superior product with exceptional quality of service.  Interact has chosen strategic partners that provide best-in-class products and services.  Our belief is that our services must be cost effective and provide ease of use, all the while allowing for future strategic benefits and technological advancement.

## Broadcast

Interact has formed a strategic partnership with a cutting edge MCVPD (multi-channel video programming distributor) that aggregated over 250 channels of linear programming and access to VOD and SVOD content utilizing the latest adaptive bitrate streaming technologies and CODECs (compression/decompression technology – HEVC/H.265) currently available.  We have established a 10-gigabit closed-loop fiber ring to distribute digital media programming that is being aggregated with nearly 680 points-of-presence.  Interact has also partnered with a global middleware provider that is also jointly developing Interact’s fCAST™ next generation casting solution.  Interact’s role is that of an Integrator overseeing sales, product implementation, tier 1/tier 2 service/support and ongoing operations.

All of our linear channels, video on-demand and streaming on-demand channels are transcoded through a Nvidia farm in HEVC/H.265 to reduce bandwidth requirements and produce the best quality streaming digital media experience possible.  Incredibly efficient streams are produced with full support for 4K resolution natively.

Transcoding is necessary to change the CODEC, the bitrate of the stream, the size of the image and create multi-bitrate streams if required. All of our streams incorporate adaptive bitrate streaming supporting both HLS (HTTP Live Streaming) and/or DASH (Dynamic Adaptive Streaming over HTTP).

Interact offers various protection methods of Content Protection which are necessary to assure secure and protected use of the content:

Tokenization

DRM (Digital Rights Management), supporting Widevine, Playready and Fairplay

Conditional Access

Geo IP Protection

## Delivery

The distribution of the channels into your network will be organized by Interact and powered by our MCVPD partner utilizing a private CDN (content distribution network).

Interact will provision a minimum synchronous 500Mbps private circuit for the distribution of Interact’s IPTV video programming.  Peer connectivity costs to the NAP or Data center, from each location, can vary based on proximity to the closest NAP.  However, LEC (Local Exchange Companies) are obligated by law to provide the connects required to 3rd parties at fixed and cost-effective rates based on the distance from exchange.

This private circuit may also be utilized to provision Internet access as well – we will work with each client to provision the most economical circuit to each location.

There are a wide variety of benefits to provisioning a private network.  The primary reason for the private CDN is so Interact can legally provide IPTV programming to each facility.  A private network also ensures quality of service and removes any third parties that may restrict bandwidth or throttle high bandwidth time-sensitive data.

Benefits of a private network:

Source Satellite Ingestion preserving the highest quality feed available for each channel

UHD/4K transcoding at our down-link facility provide the highest quality and efficient H.265/HEVC digital media streams

Provides exceptional picture quality

Provides incredible bandwidth efficiency

Full Nvidia-based, hardware-accelerated processing dramatically increases data compression

Nationwide distribution network for IP transport, saving local loop costs

Redundant 10 gigE fiber IP transport loop with 680+ peering locations across the US

Entire network is private-line/leased-line deployments (Closed Loop Requirement)

DSCP (differential services code point) support to ensure network optimization and QoS (Quality of Service) to the end point

## LAN (Local Area Network) Transport

One of the most formidable challenges for any multi-dwelling integrator is delivery of Internet via the ‘miracle mile’ to the guest room or apartment unit.  This issue has plagued integrators for years, with many forced to spend tens of thousands of dollars trenching, directional boring and rewiring to accommodate a traditional Cat 5e/6 switched Ethernet network or even retrofit with Fiber.

Interact is leveraging robust IP over Coax/UTP technology that delivers cost-effective, high-bandwidth (synchronous 1.7Gbps PHY rate), low-latency-transport technology to complete delivery of gigabit IP transport within MDUs and out-of-home markets (i.e. Hotels, Vacation Rentals, Time-Shares, Hospitals, Restaurants, etc.).  Re-using the copper to the building and/or in-building wiring eliminates costly rewiring and the resulting disruption and downtime.  G.hn offers the basis for a fully-managed plug-and-play infrastructure for Internet, IPTV and then over-encompassing Wi-Fi/PAN, IoT, and even 5G offload via Wi-Fi.

## IPTV Management

The IPTV Management System (IMS) offers content management (Live TV, Radio Channels, Movies, Series and Music), VOD Store management, Advertisements, multiple (UI) user interface templates, product management, subscription management and CRM, an IQ Analytics Reporting Engine and system management.

The IMS provides full support for individual billing (if necessary) and other used operational systems.

The IPTV Management System is fully integrated with encoding/transcoding head-end, content protection/DRM and content delivery. Furthermore, the IMS manages the Interact application that is available and fully integrated on multiple streaming-media devices.  The IMS offers near 100% uptime, unlimited scalability and backup.

Interact’s network management system and dashboard to remote manage all aspects of the WAN/LAN network allows us to remote manage all end-points of the network, provide uptime of end-point devices, accommodate dynamic VLANs (Virtual Local Area Network), PANs and provide a host of other management tools.

### Service Summary

This unique combination of technologies will enable the distribution of high-speed Internet and digital media to these multi-dwelling buildings, providing cost-effective high-speed Internet and digital media programming.

Interact provisions a private network, installs a single server and network switch and/or GAM (G.hn Access Multiplexers) at the property head-end, utilizes the existing wiring infrastructure, and provisions the client’s own streaming device, with our downloadable app (with future Interact fCAST™ capability) for the resident/guests to stream their subscribed digital media.

## Managed Network and Personal Area Network (PAN)

Interact’s most-desired implementation is when we create a single, managed Wi-Fi environment in each MDU that provides a service that mitigates the issues that occur with the ever-increasing number of connected devices on a network.  Removing overlapping and competing Wi-Fi access points, that are clearly not necessary, is an absolute must.  The solution calls for creating personal area networks (PANs) on a single Wi-Fi local area network (LAN).

A PAN is essentially a private network within a public network, utilizing software rather than dedicated Wi-Fi radios in each unit.  The rationale for PAN, utilizing an established visitor-based network (VBN) provider or establishing dynamic VLANS is that we can utilize one internal Wi-Fi network, yet allow for the perception of an individual private network for purpose of an individual resident who wants to have his/her own Wi-Fi network.  By doing this, the resident will have the ability to segment their network from their neighbors; will not see competing SSIDs (“service set identifier” or simplified – name you give your Wi-Fi access point); will not face complications from channel conflict from surrounding Wi-Fi access points; and will have the ability to create this PAN utilizing software instead of hardware.

The benefits to a managed Internet service do not just include the PAN, they also provide the ability to manage Quality of Service (QoS) for time-sensitive data (i.e. voice and video) from non-sensitive traffic.  It also offers the ability to provide the MDU owners with a network that they can utilize for IoT devices of their own, like remote door locks (public and for the resident's unit), security cameras, CO2 sensors, and noise sensors.  The owner can also add a 'skinny bundle' of linear broadcast programming to augment the resident's other streaming services, providing local affiliate programming and select cable programming.

A future benefit for the owner is to offload Wi-Fi 6/5G traffic, utilizing this same network instead of having to install costly DAS (distributed antenna system); something that also adds to interference and RF (radio frequency) interference challenges.

We intend to create a managed Internet solution that provides a basic 'managed utility' for the MDU owner's residents that they can offer in a cost-effective manner as a part of an amenities package.  Our goal is to reduce resident complaints, allow MDUs to differentiate themselves from their competition, and potentially allow MDUs to profit from this implementation.  With current trends, the need for a fourth utility providing managed Internet access services will be as routine as providing water, gas and electricity.

## MDU Owner Benefits:

Increase the overall quality of the Internet experience for Residents

  - Reducing radio interference and congestion

  - Better control over the infrastructure

    - How it is installed, maintained, utilized, supported, etc.

    - Take back control currently given to third-party entities

Provide a cost-effective amenity that can be provided at a lower cost than can be obtained otherwise

Offer a multitude of private-network benefits to an MDU owner

  - Owners who have multiple buildings can off-load voice, POS etc. to the cloud

  - Utilize network to off-load costs for vendors who may want to utilize network for 5G tower and small cell implementations

  - Improved security from data breaches/PCI compliance (Payment Card Industry compliance)

    - If utilizing POS for rent, laundry, other – may be more applicable to out-of-home markets

Negate need for expensive DAS system, removing complaints due to poor cell coverage

  - Wi-Fi 6 will allow for off-load of 5G traffic to the Internet

Provide added security and energy management within building

  - Manage public and resident door access

  - Install Wi-Fi security cameras throughout facility

  - Ability to install IoT devices such as CO2 sensors, noise sensors, thermostats/EMS (energy management systems), remote door access, and other IoT devices

Provide Secure Personal Area Networks (PANs)

  - Client-to-client control and visibility of connected devices limited to each PAN

  - Offer as part of amenities package/build value to other areas such as work-areas, day rooms, exercise room, pool etc.

  - Cell phone coverage for Wi-Fi dialing throughout facility – public areas, hallways etc.

  - No need to manage multiple vendors wanting access to facility

Better understanding of how the infrastructure is utilized – enhance ability to understand your residents

## Resident Benefits:

Improved Internet bandwidth, television, Wi-Fi and IoT experience

More economical television offering – targeting $10 to $50 basic/loaded ‘skinny bundle’ offerings (AT&T TV starts at $49/month, YouTube TV is $60/month)

  - Skinny bundle to provide local programming, sports and desired linear channels

  - Provide ability to subscribe to packages and a-la-carte offerings (as allowed by programmers)

  - Support via provided STB, downloadable AppleTV/Amazon TV App or download iOS or Android on phone/tablet

  - Provide 72-hour catch-up TV along with DVR service

Instant service availability with simple resident self-installation

Parental control with 'white listing’ services

Access to IoT devices provides increased level of security for residents (access to cameras/door locks)

  - Residents can install a variety of their own IoT devices and gain access to network without fear of interference from surrounding neighbors’ units

Seamless Internet roaming throughout the building

5G cell coverage within and surrounding building

## Interact Advantages

Private networks to each MDU with robust Internet bandwidth to stream linear programming, gigabit Ethernet and VOD (subscription and other) programming

  - Provide cost savings for MDU owners who have several buildings

    - Eliminate PCO (Private Cable Operator) requirement – not crossing a public right-of-way

    - Can layer other cloud services onto this private network – voice, energy management, security etc.

  - Allow for off-loading of 5G traffic over Wi-Fi (and the underlying fixed broadband infrastructure)

    - Utilize roof-top for small-cell 5G towers

    - Bundle with ISP for Internet bandwidth redundancy or replace for cost reduction

Cost effective and robust wired deployment in the building

  - Leverage existing Coaxial and/or UTP wiring in the building.  Absolutely no need to rewire the building!

  - Utilize best-of-breed G.hn technology to provide synchronous 1.7Gbps PHY rate to each unit/room

    - One 24-port switch can support up to 384 units (up to 16 ports per head-end switch port)

    - Switch ports are configurable (1,4, 8, 12 and 24 port) to accommodate smaller properties

  - Provide client switch which has full layer-2 support with up to two 1Gbp Ethernet ports – costs comparable to other CPE devices

    - Future cost reductions by integrating G.hn into STB/SmartTV and other IoT hardware

    - Installation is quick, simple and not disruptive

  - Provide IP to WAPs strategically located, utilizing either UTP or coax in unit/room

  - Remove significant digital media streaming traffic from Wi-Fi network

  - Seamlessly overlay on existing legacy cable providers, ensuring limited disruption to existing provisioning during transition period

Create secured HSIA network for each resident within building

  - Provide managed HSIA service for each resident

  - MDU owner provides bulk Internet access and provides secured PAN

    - Seamless PAN roaming throughout property (patent pending)

    - Client-to-client control and visibility of connected devices limited to each PAN

    - Parental control with 'white listing’ services

  - Maximize bandwidth for each resident (allow ability to upsell tiered bandwidth)

  - Reduce cost and increase up-time by utilizing managed and unmanaged ISPs

  - Other potential solutions being contemplated for robust WAN circuit provisioning

# G.hn – IP over Copper/Coax (IPoC)

IP over Copper/Coax has significantly evolved over the last ten to fifteen years.  The IPoC solution we believe to be relevant and necessary for Interact is an international telecommunication standard (ITU-T) called G.hn.  Interact has partnered with a world leader in distributing IP over existing copper or coaxial infrastructure. They provide the most cost-effective solution to deliver gigabit Ethernet throughout existing buildings without the cost or hassle of pulling new coaxial, unshielded twisted pair or fiber. This has saved their customers millions of dollars and provided them with full layer 2+ network functionality over both coax and twisted pair.

A few advantages of utilizing G.hn in a brown-field implementation:

No re-cabling

Achieve Fiber-equivalent bandwidth with a future-proof solution

No construction work required

No disruption or downtime

Zero-touch provisioning by subscriber

Subscriber self-installation – automated activation via captive portal

No service interruption

Proven applications

Next-generation PHY rate of 5Gbps over copper and 10 Gbps over coax with the existing infrastructure – coming 2022

Many may think of DOCSIS (Data Over Cable Service Interface Specification) when they discuss IPoC. However, DOCSIS technology was developed by Cable Labs (which caters to the cable MSOs).  The early DOCSIS implementations were very expensive, worked only on coaxial cable and did not offer required network management components – they were considered substandard to the capabilities of traditional switched Ethernet networks. In addition to DOCSIS, MoCa was also an IPoC solution that emerged and was used primarily by DirecTV; also considered to be incomparable to a switched-ethernet network.

Most recently, G.fast was touted as the solution for delivery of high speed over legacy cabling in brown-field scenarios, however, the lofty goals set for G.fast have yet to materialize and despite numerous announcements, many operators have quietly reduced or abandoned their plans to deploy G.fast.

The following factors promote G.hn, from an MDU owner point of view:

G.hn makes better use of coaxial cabling than G.fast, DOCSIS and MoCa, providing better/faster Internet access to all residents

Unlike DOCSIS, G.hn does not require amplifiers

Unlike MoCa, G.hn works over existing telephone wiring – MoCa only supports coaxial plants

Unlike G.fast, G.hn is highly scalable and supports double the number of subscribers as G.fast

Unlike G.fast, G.hn supports a one-to-many configuration in a coaxial cable environment

## Applications

G.hn is ideal for retrofitting existing buildings with world-class, metro-grade, high-speed IP networking equipment.  Any building with existing coaxial or UTP cabling is eligible for this easy plug-and-play IP service transport solution.

Interact provides a suite of provisioning and configuration tools enabling a fully managed network service for MDUs.  The configuration tools provide the ability to deliver subscriber bandwidth profiles and fully segmented VLAN configuration providing building wide personal area networks (PANs) for residents via a meshed wireless network.

Near future applications will include: 5G offload, enabling a seamless low-latency high-bandwidth user experience for 5G-enabled residents and IoT optimization enabling the full spectrum of wireless device-based services.

In summary, the G.hn will allow Interact to provide a fully managed solution required by all smart building technology.

## Distance limitations

For both scenarios, UTP and coaxial cable, G.hn operates in the 5-205MHz range. We can utilize a CATV filter to clear spectrum from channel 2-23 (54 to 206HHz) allowing more than one service provider on the same coaxial cable.  The limited bandwidth spectrum (5-205 MHz) on both UTP and coaxial cable allows for reduced attenuation, which in turn allows high-speed Internet to run over longer distances.

Refer below for distance limitations based on physical legacy wiring:

1-pair copper/UTP (SISO)	– 	GigE to 450 feet

2-pair copper/UTP (MIMO)	– 	GigE to 800 feet

RG59	– 	GigE to 1000 feet

RG6	– 	GigE to 2600 feet

If you use a value of 15-20 feet vertical between floors and then 50-100 feet for the horizontal coverage (per floor), you get very solid gigabit Ethernet throughput for MDUs of almost any size.

# A Deeper Technology Review

## About G.hn Technology

The G.hn standard was developed under the International Telecommunication Union’s Standardization arm (ITU-T) and promoted by the HomeGrid Forum along with global telecom carriers, semiconductor companies, system vendors, academic institutes. ITU-T Recommendation (the ITU’s term for standard) G.9960 received approval on October 9, 2009, which specified the physical layers and the architecture of the standard. The Data Link Layer (Recommendation G.9961) was approved on June 11, 2010. The ITU-T extended the technology with multiple-input, multiple-output (MIMO) technology to increase data rates and signaling distance. This new feature was approved in March 2012 under G.9963 Recommendation.

The ITU-T G.9960 G.hn Wave-2 standard leverages existing telephone wiring (UTP, Cat 3 or Cat 5/5e) or coaxial cabling (RG-6/RG-59) to deliver gigabit Ethernet service to each subscriber inside an MDU without the cost, complexity and delays associated with in-building fiber installation.’

G.hn allows operators to simplify the management of their access networks with an Ethernet-bridge technology that is highly scalable and less complex than DSL-related technologies. With G.hn, operators deliver advanced services such as gigabit high-speed residential Internet and 4K IPTV without the high capital and operational expenses associated with a fiber retrofit. Each G.hn subscriber port supports up to 1.7 Gbps of dynamically allocated bandwidth for near-symmetrical gigabit services over existing telephone wire or coaxial cable. In addition to delivering residential high-speed Internet, the G.hn Access Multiplexer (GAM) solution complies with MEF CE 2.0 and is ideally suited to deliver gigabit Internet services in an MDU deployment.

G.hn Wave-2 is widely used by large operators in multiple markets with a strong base in Southeast Asia, where it provides gigabit services to MDUs of all sizes and for residential neighborhoods. Operators such as Korea Telecom leverage the robustness of G.hn and its predictable bandwidth to scale to large deployments and complement PON (Passive Optical Network) infrastructure investments.

G.hn is also a perfect fit for coaxial cable networks, which typically have a point-to-multipoint topology, providing a solution capable of delivering gigabit services at a fraction of the cost of traditional DOCSIS solutions that rely on expensive remote PHY or remote MAC CMTS equipment.

## UTP compatible cable types

The Twisted Pair product line is compatible with any single or multi-pair unshielded twisted pair cabling. Some common cable types include:

CAT1 to CAT5, CAT5e, CAT6, & CAT7

Nano-skew

Belden twisted pair - Audio, Control and Instrumentation cable

Crestron Cresnet

## Coaxial compatible cable types

Product line is compatible with all common coaxial cabling. Some standard cable types include:

RG58, RG59, RG6, RG11

Mini-coax

## Advantages over G.fast

G.fast’s DPU (distribution point unit)/switch has 16 ports, which raises two major concerns:

  - Delivery of true gigabit Internet speeds on current-generation DPUs requires the bonding of two G.fast ports, reducing by half the number of subscribers the DPU can serve

Effectively doubling the cost per port vs. G.hn

  - It is not possible to add another small DPU when the ports of the first DPU are all in use, due to noise cancellation/vectoring processor in the DPU

    - The DPU cannot work alongside another DPU

    - Must go through a ‘rip and replace’ process to install a larger (and more expensive) DPU.

With the typical coaxial cable infrastructure being point-to-multipoint, to support cable TV and DOCSIS, it becomes nearly impossible to use G.fast because it is strictly a point-to-point technology.

  - G.hn doesn’t have these problems. Each port on the G.hn solution can operate as a single input, single output (SISO) single-pair or multiple input, multiple output (MIMO) two-pair port on copper.

  - The G.hn VectorBoost technology for copper pairs works from the get-go across multiple G.hn GAMs for seamless scalability

  - G.hn leverages the point-to-multipoint nature of coaxial cabling to deliver gigabit Ethernet

## Advantages over DOCSIS

G.hn provides symmetric upstream and downstream data rates and utilizes dynamic bandwidth allocation to deliver the ideal bandwidth to each customer on the network.

  - G.hn offers over 1.7Gbps real-world data throughput rates (next gen G.hn chipsets will support up to 10Gbps PHY rate over coaxial cable)

  - DOCSIS 3.0 tops out at 1.2Gbps real-world downstream and upstream capacity is capped at 200 Mbps in a shared medium

G.hn products provide the upstream and downstream bandwidth that customers require without the massive capital investment required to install a DOCSIS CMTS headend.

## Advantages over MoCa

MoCA can only deliver Ethernet over coaxial cable and is unable to operate over phone line/ unshielded twisted pair.

  - G.hn provides a universal solution for delivering gigabit Ethernet over any existing copper infrastructure.

MoCa does support a ‘one-to-many’ configuration but has drawbacks that render the technology pretty much unusable for an MDU or Out-of-Home market.

  - MoCa utilizes 1GHz of spectrum which attenuates very quickly, rendering it unusable

  - MoCa is limited to a 10dB signal loss.

  - G.hn can absorb up to 40dB signal loss and still provide a PHY rate of 1.8Gbps.

MoCa 2.5 does tout 2.5Gbps speeds in optimal conditions, but because of 1 GHz spectrum, technology is really restricted for use inside a home as a LAN extension.

  - G.hn utilizes 5MHz to 205MHz spectrum.

Source – Muvi, ‘50% of US OTT homes subscribe to multiple services’

Source eMarketer – ‘Partnerships Accelerates Despite OTT Exodus from PayTV’, July23, 2018

Cord Cutters News, WSJ – Ampere Analysis